# Vigor 2767 DrayTek Product Details

# Overview

The DrayTek Vigor 2767 series a high performance broadband router suitable for home offices or scaled to SME businesses requiring a secure and reliable connection for remote access and/or remote working for cloud applications, VoIP, and remote working. The 2767 supports both a traditional DSL broadband (VDSL2 and ADSL2+) and modern fibre, making it suitable for VDSL/ADSL connections, offering both primary and backup WAN failover options.

The Vigor 2767 provides advanced security and networking capabilities including a firewall, content filtering, and support for modern VPN protocols such as IPsec, OpenVPN, and WireGuard.

The wireless edition (Vigor 2767ax) includes dual-band Wi-Fi 6 (AX3000) connectivity, enabling fast wireless performance.

If wanted, you can also access and manage the router through DrayTek’s VigorACS platform and can manage compatible DrayTek access points and switches.

Different models cover wired, Wi-Fi6 and integrated 4G/LTE versions to suit your needs. These include:

- Vigor 2767: wired VDSL / Ethernet router

- Vigor 2767ax: Wi-Fi 6 dual-band router

- Vigor 2767ax-4G: Wi-Fi 6 router with integrated 4G/LTE modem

# Features

Shortened features (max 20 lines)

General

- Integrated VDSL2 / ADSL2+ DSL modem with Multi-WAN support with failover options

- Ports include: one DSL WAN port, one 2.5GbE switchable WAN/LAN port, three Gigabit Ethernet LAN ports and an optional USB mobile broadband WAN support on certain models

- Remote worker and LAN-to-LAN VPN support with up to 16 concurrent VPN tunnels

- WPA2/WPA3 wireless encryption, plus an SPI firewall with DoS protection

- Dimensions: 207 × 131 × 42 mm

- Weight: 455 g (wired model) or 506 g (wireless models)

- Wall-mountable design

- External power supply: 12 V DC

Wireless (AX models)

- Dual-band Wi-Fi 6 (802.11ax)

- AX3000 wireless performance

- 2.4 GHz and 5 GHz bands

- Up to 8 SSIDs (4 per radio)

Centralised Management

- Compatible with VigorACS network management platform

- Virtual AP controller supporting up to 20 VigorAP access points

- Virtual switch controller supporting up to 5 VigorSwitch devices

- DrayTek Mesh support (compatible models)

END

Connectivity

- Integrated VDSL2 / ADSL2+ DSL modem

- 1 × DSL WAN port

- 1 × 2.5GbE switchable WAN/LAN port

- 3 × Gigabit Ethernet LAN ports

- Multi-WAN support with failover options

- Optional USB mobile broadband WAN support on certain models

Wireless (AX models)

- Dual-band Wi-Fi 6 (802.11ax)

- AX3000 wireless performance

- 2.4 GHz and 5 GHz bands

- Up to 8 SSIDs (4 per radio)

Performance

- Up to 2.3 Gbps firewall throughput

- Up to 300 Mbps IPsec VPN throughput

- Up to 16 concurrent VPN tunnels

- Up to 50,000 NAT sessions

VPN Features

- Supported VPN protocols:

  - IPsec

  - OpenVPN

  - WireGuard

- Remote worker and LAN-to-LAN VPN support

- Hardware-accelerated encryption

Network Features

- Stateful Packet Inspection (SPI) firewall

- Web content filtering

- Quality of Service (QoS) and bandwidth management

- VLAN support (IEEE 802.1Q)

- Multi-WAN load balancing and failover

- Port forwarding, DMZ, and UPnP

- Dynamic DNS including DrayDDNS

Routing and Protocols

- IPv4 and IPv6 support

- Routing protocols including:

  - RIP-1 / RIP-2

  - OSPF

  - BGP

Security

- WPA2/WPA3 wireless encryption

- SPI firewall with DoS protection

- RADIUS and certificate authentication support

- Multiple encryption standards including AES-256

Centralised Management

- Compatible with VigorACS network management platform

- Virtual AP controller supporting up to 20 VigorAP access points

- Virtual switch controller supporting up to 5 VigorSwitch devices

- DrayTek Mesh support (compatible models)

Physical Specifications

- Dimensions: 207 × 131 × 42 mm

- Weight:

  - 455 g (wired model)

  - 506 g (wireless models)

- Operating temperature: 0°C – 40°C

- Humidity: 10–90% non-condensing

- Wall-mountable design

Power

- External power supply: 12 V DC

- Input: 100–240 V AC

- Power consumption:

  - ~13.7 W (wired models)

  - ~21.6 W (Wi-Fi models)

Compliance

- CE, UKCA, RoHS compliant

- Standard 2-year warranty

# Ordering information

NA

# Accessories

- Vigor 2767 series router

- Quick start guide

- Cat-5e RJ-45 Ethernet cable

- RJ-11 DSL cable

- 12 V DC power supply with UK plug

- Detachable Wi-Fi antennas (wireless models only)

# Support and downloads

- Data Sheet

- User guide